# Response to the RESCUE specification

Developer Saeed Nik    Software release 3.0.1

# 1 Context and objective

We implemented RESCUE for fixed-wing ArduPlane 4.6.3. Detection and rescue control run inside the firmware. QGroundControl and the local panel operate the simulator and display the evidence.

Section 3 permits necessary additional settings when their purpose is explained in the README. We retained the five required parameters and added settings for decisions whose numerical limits were not specified.

Early tests could confuse takeoff level-off and short maneuvers with falling. To implement sustained conditions in section 4.1 and prevent repeated transitions under section 5, we added measured-descent confirmation and hysteresis.

Section 4.4 requires stable level flight; scenario 6 in section 7 requires safe degradation when recovery fails. We therefore defined stability limits and progress deadlines. Full throttle does not prove a climb: increased airspeed can also represent useful energy recovery.

One recorded steep descent reached about 30 m Home while downward acceleration was near zero, so the acceleration rule did not trigger. We added a steady-descent safeguard in support of the rescue objective in section 1. This explicitly extends the acceleration condition in section 4.2.

QGC throttle movement without a mode change was not recognized as pilot acknowledgment. Section 4.4 specifically requires a mode change; deliberate stick acknowledgment is an optional extension. Charts and replay support the log review in sections 5 and 6; a graphical panel was not explicitly required.

# 2 General behavior

| State | Implemented behavior |
| --- | --- |
| MONITOR | Observe an armed, flying aircraft without changing its current mode. |
| WARNING | Below 200 m Home with confirmed danger, send WARNING messages at about 1 Hz. |
| RESCUE | Below 100 m with continued danger, capture control and recover toward 200 m using TECS. |
| STABILIZED | After 2 s of continuous stability, signal stabilization and start the 5 s pilot window. |
| Handover | Accept a valid pilot mode change; optional deliberate stick input selects FBWA. |
| Parachute | No response or failed recovery requests AP_Parachute, motor cut and disarm. |

# 3 Parameters

This implementation adds 26 RSC parameters: the five specified settings and 21 justified additions. These tables contain only those custom parameters. Values are the tested profile values; RSC_ENABLE defaults to 0 in the firmware and is set to 1 for the supplied tests.

| Parameter | Value | Purpose |
| --- | --- | --- |
| RSC_ENABLE | 0 / 1 | Enables all rescue logic. Firmware default: 0; supplied test profile: 1. |
| RSC_ALT_WARN | 200 m | Warning boundary and recovery target, relative to Home. |
| RSC_ALT_TRIG | 100 m | Automatic capture boundary during continued qualifying descent, relative to Home. |
| RSC_ACC_Z | 5 m/s² | Gravity-compensated downward acceleration threshold for the original entry rule. |
| RSC_HAND_S | 5 s | Pilot response window, starting only after stable recovery is confirmed. |
| RSC_DWELL_S | 0.2 s | Condition dwell after the fixed gate of at least 1 s descent and 2 m height loss. |
| RSC_ALT_HYST | 10 m | Height margin for clearing a warning without chatter near the boundary. |
| RSC_SINK | 5 m/s | Sink threshold for the additional impact-entry path. Sink alone does not trigger it. |
| RSC_UN_ANG | 60° | Severe roll/pitch diagnostic and recovery-failure threshold. |
| RSC_UN_RATE | 90°/s | Severe rotation threshold used to assess instability during recovery. |
| RSC_TTC_S | 6 s | Time-to-Home margin for the additional low-height entry path. |
| RSC_STAB_S | 2 s | Continuous duration for which every stable-flight criterion must hold. |
| RSC_ALT_TOL | 5 m | Height tolerance around the target: 195–205 m for a 200 m target. |

The first five rows are direct requirements. Additional settings define persistence, warning clearance, stability and failure handling. RSC_SINK and RSC_TTC_S belong to the added low-height safeguard. Attitude or rotation alone does not capture control.

# Additional parameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| RSC_VZ_TOL | 1 m/s | Maximum absolute vertical speed accepted as stable level flight. |
| RSC_ANG_MAX | 10° | Maximum absolute roll and pitch accepted as stable. |
| RSC_ST_RATE | 15°/s | Maximum rotation rate. A brief zero-angle crossing does not count as stability. |
| RSC_RECOV_S | 90 s | Overall deadline for stable recovery, measured from capture. |
| RSC_FAIL_S | 3 s | Allowed persistent excessive sink or severe instability after the initial grace period. |
| RSC_MIN_ALT | 30 m | Low-height recovery limit. Below it, the extra entry path accepts sink above 2 m/s. |
| RSC_PROG_S | 10 s | Window for assessing actual height or total-energy-height progress. |
| RSC_PROG_M | 3 m | Minimum height gain OR energy gain equivalent to 3 m of height in that window. |
| RSC_IMPACT_S | 2 s | Urgent predicted Home-datum crossing threshold while sink exceeds 2 m/s. |
| RSC_REARM_S | 10 s | Post-handover cooldown plus a clear-risk interval; imminent danger can override it. |
| RSC_STK_EN | 1 | Optional deliberate stick/throttle acknowledgment. Set 0 for mode-change-only handover. |
| RSC_STK_US | 120 µs | Minimum fresh raw-input change from the stabilization baseline. |
| RSC_STK_S | 0.3 s | Duration of valid fresh deliberate input before handover to FBWA. |

Section 3 allows necessary settings with a README justification. The added values make continued falling, stable flight, failed recovery and pilot acknowledgment testable. They supervise the existing controllers; they do not replace TECS.

Full throttle can initially increase speed rather than height. The progress check therefore accepts either a height gain or an equivalent total-energy-height gain. A low-speed aircraft is not declared unrecoverable merely because it first needs to accelerate. Existing TECS, airspeed and parachute settings remain in the complete parameter file; they are not custom RSC parameters.

# 4 Detailed phase requirements

## 4 1 Monitor to warning

The 50 Hz supervisor uses gravity-compensated downward acceleration in the earth frame, filtered at 5 Hz. Raw body acceleration and normal upward takeoff acceleration are not treated as falling. Descent must exceed 0.5 m/s continuously for at least 1 s and produce at least 2 m of measured height loss.

After descent confirmation, downward acceleration must exceed RSC_ACC_Z for RSC_DWELL_S. Below RSC_ALT_WARN, WARNING messages are sent at approximately 1 Hz without changing mode. When the condition clears, temporal and height margins prevent warning chatter.

## 4 2 Warning to RESCUE

The original section 4.2 path requires confirmed continued descent, downward acceleration above 5 m/s² and height below 100 m Home. It enters mode 26 RESCUE from the current mode. The aircraft must be armed, flying and have a usable parachute configuration. TAKEOFF remains protected; false capture was corrected in detection rather than hidden by excluding that mode.

The added steady-descent path uses the same 1 s / 2 m confirmation. Height must be below 100 m, sink above 5 m/s, and estimated time to the Home datum below 6 s. Below 30 m, sink above 2 m/s is accepted while the time condition remains. The condition must persist for 0.2 s. Risk bit 32 and the predicted-impact message distinguish this entry path.

The recorded fault reached 30.06 m Home with 7.67 m/s sink and about -0.034 m/s² downward acceleration. No sample satisfied the acceleration gate during that descent. The new path deliberately extends the literal specification. Shallow descent or low height alone is insufficient. An intentional dive into the same danger envelope can also activate protection.

The extra path yields to actual braking only when both gravity-compensated acceleration and the filtered sink trend show upward deceleration, with a predicted stop more than 5 m above Home. A physically responding pull-out may receive one grace period of at most 0.5 s per descent, only above 30 m with more than 2 s remaining. Repeated stick pulses cannot extend it. Neither braking clearance nor grace suppresses the original acceleration rule.

## 4 3 Recovery

The height target is RSC_ALT_WARN. Existing TECS manages height and airspeed; existing attitude controllers level the wings with a zero-roll target. During the main climb, the highest available throttle is requested within the existing output, power and slew limits. Near the target, TECS adjusts throttle to settle. No separate recovery pitch or throttle control law was written.

A near-vertical entry could leave the internal TECS pitch-demand slew state far outside the usable demand. Emergency entry seeds that existing state within the configured pitch limits, avoiding unnecessary delay. TECS energy control, speed protection and anti-stall behavior remain active.

## 4 4 Stabilization and handover

| Criterion | Required stable condition |
| --- | --- |
| Height | 200 m Home with a tolerance of ±5 m. |
| Vertical speed | Absolute value no greater than 1 m/s. |
| Attitude | Absolute roll and pitch no greater than 10°. |
| Rotation | Body rate no greater than 15°/s. |
| Airspeed | At least the existing minimum; 10 m/s in this profile. |
| Persistence | Every criterion must hold continuously for 2 s. |

Once all criteria hold, the stabilized message and five-second timer begin. A valid pilot mode selection, including FBWA, is accepted. Losing stability during handover reactivates recovery assessment.

With optional RSC_STK_EN=1, valid fresh stick or throttle input must change by at least 120 µs from the stabilization baseline and persist for 0.3 s; handover then selects FBWA. Unchanged held input, noise and stale messages do not count. Setting 0 retains the mode-change-only behavior explicitly required by section 4.4.

## 4 5 Parachute

If RSC_HAND_S expires without a valid response, AP_Parachute is requested, the motor is cut and the aircraft disarms. In the test profile, the release output activates after 0.5 s and modeled canopy inflation takes 1.5 s. Start a fresh simulation to test again with a packed canopy.

# 5 Edge cases and safety

| Issue | Decision and implemented behavior |
| --- | --- |
| Capture from MANUAL | The supervisor is mode-independent. After stable recovery the pilot can select a mode during the handover window. |
| Repeated transitions | After handover, apply 10 s cooldown and require a clear-risk interval. The extra imminent-danger path may override cooldown. |
| Disabled supervisor | RSC_ENABLE=0 produces no supervisor intervention; checked against the original disabled-case BIN. |
| Decision logging | RSC records height, acceleration, sink, attitude, airspeed and risk bits. MODE, TECS, RCOU and ARM provide independent evidence. |
| Normal takeoff or brief descent | A transient below 1 s or less than 2 m measured loss cannot meet the descent gate. Normal 40/80 m flights and short transients had no false capture. |
| Full power without a climb | Over a 10 s window, require at least 3 m height gain OR equivalent total-energy-height gain; airspeed recovery can count. |
| Continued sink or severe rotation | After the initial 2 s recovery grace, sink above 3 m/s or severe instability persisting for 3 s can request the parachute. |
| Urgent or prolonged failure | Low height below 30 m, urgent predicted Home crossing, invalid navigation data or the 90 s overall deadline can terminate recovery. |

Stability uses angles, rotation, airspeed and persistence together. Crossing zero roll briefly during a full rotation does not establish stable level flight. Severe attitude alone does not trigger initial capture.

Steady-descent protection, bounded pull-out grace, stick acknowledgment, progressive canopy modeling and the reporting panel are disclosed additions. Home-relative height is not measured terrain clearance. Rising terrain, estimate delay, a very late dive or limited aircraft performance can still make rescue impossible.

Actual QGC virtual-joystick trials are recorded separately from repeatable MAVLink-driven tests. The complete ten-case attended QGC campaign remains incomplete and is not reported as passed.

# 6 Deliverables

| Required item | Supplied file or folder |
| --- | --- |
| Firmware patch | 01_Patch/rescue.patch, 30 modified source files, BUILD.md and build manifest. |
| Complete parameter file | 02_Parameters/rescue.parm and the simulation site reference. |
| Compiled SITL firmware | 03_SITL/bin/arduplane.exe with DLLs, Python dependencies, panel and both executable launchers. |
| BIN and short phase analysis | 04_Logs/bin contains eight original acceptance BINs; Phase_Analysis.md identifies each transition. |
| README with decisions | 05_README/README.md documents the custom parameters and decisions; this English response is included alongside it. |

The English delivery uses the same verified 3.0.1 flight firmware. Its presentation adds recorded videos and native-data charts. SHA-256 receipts identify the exact firmware, patch, parameters and original evidence.

| Phase in no-response flight | Native simulation time | Home height |
| --- | --- | --- |
| Warning | 68.384 s | 141.47 m |
| Control capture | 71.925 s | 99.96 m |
| Stabilized | 101.905 s | 197.29 m |
| Parachute request | 106.925 s | 199.90 m |

In this flight, capture occurred at 99.96 m with 7.42 m/s² downward acceleration. Stabilization was confirmed at 197.29 m, followed about 5.02 s later by the parachute request. RCOU and ARM verify motor cut and disarm. The table shows supervisor request time; actuator output follows its configured delay.

https://github.com/saeednikakhtar/rescue-flight-lab

# 7 SITL scenarios

All seven required scenarios passed on the supplied binary. Scenario 2 was flown separately from MANUAL and AUTO, so the acceptance set contains eight flights. Results are checked against the original BIN using RSC, MODE, TECS, RCOU and ARM.

| No. | Required scenario | Recorded result |
| --- | --- | --- |
| 1 | Induced dive below warning height | WARNING below 200 m with qualifying acceleration; no forced mode change or control capture. |
| 2 | Continued falling below capture height | Automatic capture from MANUAL at 99.52 m and AUTO at 99.71 m; both exceeded 5 m/s² down acceleration. |
| 3 | Recovery and stable level flight | Existing TECS reached the 200 m target band and met stability criteria without crossing the model minimum airspeed. |
| 4 | Pilot selects FBWA within 5 s | The post-stabilization mode change was accepted. The aircraft remained armed and the parachute stayed packed. |
| 5 | No pilot response for more than 5 s | AP_Parachute request, release output, motor cut and disarm were confirmed. |
| 6 | Unable to climb or still losing height | Engine failure was induced after capture. Insufficient progress requested the parachute at 73.54 m Home. |
| 7 | RSC_ENABLE=0 | No rescue capture, warning or parachute action attributable to this supervisor was recorded. |

Fourteen further native regressions cover ordinary takeoff and flight, brief transients, controlled descent, steady steep descent, successful and abandoned pull-outs, banked descent and engine failure. They are engineering coverage, not extra acceptance choices in the panel. One controlled-descent attempt reached the test harness approach timeout; a repeat with a longer allowance passed without a firmware change. Both records are retained.

Videos replay recorded simulator data. A separate repeat of the no-response scenario continues through full modeled canopy inflation and model ground contact. It does not increase the seven-scenario or eight-flight acceptance count.

low-height-verification.json identifies the binary and checked conditions. Results apply to the supplied native model; they are not a physical-controller, real-parachute or variable-terrain qualification.